DIGITAL SYSTEM DESIGN LABORATORY

LAB 3

IMPLEMENTATION OF BASIC COMBINATION LOGIC CIRCUIT USING VHDL

I. LAB OBJECTIVES

This Lab experiments are intended to implement Basic Combination Logic and Sequential Circuit in VHDL. Students are require to write test bench to simulate the given example code and Top level module to implement these codes in DE2-115 FPGA Kit.

a) For each experiment write the VHDL Code in three method ( dataflow, behavior and gate level)

b) For each model, write the Top Level VHDL Code to implement the these modules  in DE2-FPGA Kit

(Show implementation results in Lab report)

c) Analyze the FPGA implementation results for these model

II. PROCEDURE

II.1  LAB EXPERIMENT 1 :  WRITE VHDL CODE TO IMPLEMENT ALL LOGIC GATES  IN FPGA KIT.

II.2 LAB EXPERIMENT 2 : WRITE VHDL CODE TO IMPLEMENT THE HALF ADDER CIRCUIT IN FPGA KIT:

II.3 EXPERIMENT 3: WRITE VHDL CODE TO SIMULATE AND IMPLEMENT THE FULL ADDER CIRCUIT IN FPGA KIT:

II.4 EXPERIMENT 4 :WRITE VHDL CODE TO IMPLEMENT  2:1 MULTIPLEXER CIRCUIT IN FPGA KIT :

II.5 EXPERIMENT 5 : WRITE VHDL CODES TO IMPLEMENT  4:1 MULTIPLEXER CIRCUIT IN FPGA KIT:

II.6 EXPERIMENT 6 : WRITE VHDL CODE TO IMPLEMENT  2 to 4  DECODER CIRCUIT IN FPGA KIT:

II.7 EXPERIMENT 7 : WRITE VHDL CODE TO IMPLEMENT  8 TO 3 ENCODER WITH PRIORITY CIRCUIT IN FPGA KIT:

II.8 EXPERIMENT 8 : WRITE VHDL CODES TO IMPLEMENT  1 TO 8 DEMULTIPLEXER CIRCUIT IN FPGA KIT:

II.9 EXPERIMENT 9 : WRITE VHDL CODES TO IMPLEMENT  N-BIT COMPARATOR CIRCUIT IN FPGA KIT WITH FOLLOWING VHDL REFERENED CODE

library ieee;

use ieee.std_logic_1164.all;

entity Comparator is

generic(n: natural :=4);

port(	A:	in std_logic_vector(n-1 downto 0);

B:	in std_logic_vector(n-1 downto 0);

less:		out std_logic;

equal:		out std_logic;

greater:	out std_logic

);

end Comparator;

architecture behv of Comparator is

begin

process(A,B)

begin

if (A>B) then

less <= '0';

equal <= '0';

greater <= '1';

elsif (A=B) then

less <= '0';

equal <= '1';

greater <= '0';

else

less <= '1';

equal <= '0';

greater <= '0';

end if;

end process;

end behv;

II.10 EXPERIMENT 10 : WRITE VHDL CODES TO IMPLEMENT  N-BIT ALU CIRCUIT IN FPGA KIT WITH FOLLOWING VHDL REFERENED CODE

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_unsigned.all;

use ieee.std_logic_arith.all;

entity ALU is

generic(N: natural :=4);

port(	A:	in std_logic_vector(N-1 downto 0);

B:	in std_logic_vector(N-1 downto 0);

Op:	in std_logic_vector(2 downto 0);

Res:	out std_logic_vector(N-1 downto 0)

);

end ALU;

architecture behv of ALU is

begin

process(A,B,Op)

begin

case Op is

when "000" =>

Res <= A + B;

when "001" =>

Res <= A - B;

when "010" =>

Res <= not A ;

when "011" =>

Res <= not (A and B);

when “100” =>

Res <= not (A or B);

when “101” =>

Res <= A and B;

when “110” =>

Res <= A or B;

when “111” =>

Res <= A exor B;

end case;

end process;

end behv;

IV. LAB REPORT GUIDELINES

Students write up a report on the Verilog HDL implementation experiment projects created in this lab. The lab report should include Circuit Schematics, Truth Table, Verilog Module Codes, Verilog test bench codes, Top level module to implement the required circuit in FPGA KIT and evidences of data output evidences to validate the experiments  (The Captured Screens, Photo of FPGA Kit implementation results).